Thursday, July 30, 2026

# NEW ProMinent ProMix S-Series Polymer Blending Systems

(available NOW for special list price of $6,500 less 25% distributor discount):

The ProMix S-Series are complete activation and dosing systems for concentrated liquid polymer.

The ProMix is a pre-engineered polymer mixing system made for the water and wastewater markets. Designed as an in-line unit, the ProMix can be customized to meet most liquid polymer applications utilizing tubing pump technologies.

- Neat polymer and water are metered into the specially designed mixing chamber using a peristaltic pump in the range of 0.15 to 3.5 gph

- True Multi-Zone mixing chamber provides proper shear and mixing energy required for efficient activation of polymers

- Incoming water pressure supplies the motive force to deliver the active polymer solution to the point of feed

- Various control packages available to automate external controls signals response

- Flexibility to meet the optimum activation profile for whichever polymer you choose

- Available in a range of sizes to deliver the dosage and dilution requirements for most applications

- Please allow 2-3 week delivery time

Applications:

- Emulsion or Dispersion

- polymer activation

- Coagulant or Solution polymer feed

- Water and Wastewater treatment

- Clarification

- Sludge Dewatering

Specifications:

- Water Inlet: 3/4” FNPT

- Polymer Inlet: 3/8” Tube

- Product Outlet: 3/4” FNPT

- Max. Chamber Pressure: 37 PSIG

- Operating Pressure: 15 PSIG

- Power Supply: 120 VAC, 1 Phase, 60Hz

- Current Load: 12 Amp

- Drain Connection: 1/2”

Technical Data:

| ProMix/S-Series | Primary | Primary | Secondary | Secondary | Peristaltic |  |
| --- | --- | --- | --- | --- | --- | --- |
| P/N | Model Number | Dilution (GPH) | Rotometer(GPM) | Dilution (GPH) | Rotometer(GPM) | Pump (GPH) |
| 7746334 | 60-0.15T | 30 | 1 | - | - | 0.15 |
| 7746335 | 60-0.3T | 30 | 1 | - | - | 0.31 |
| 7746336 | 60x2-0.3T | 60 | 1 | 60 | 1 | 0.31 |
| 7746337 | 120x2-0.7T | 120 | 2 | 120 | 2 | 0.79 |
| 7746338 | 120x2-1.0T | 120 | 2 | 120 | 2 | 1.5 |
| 7746339 | 300x2-1.0T | 300 | 5 | 300 | 5 | 1.5 |
| 7746340 | 300x2-3.0T | 300 | 5 | 300 | 5 | 3.5 |