Browser Manual

ProMinent® ProMtrac

Cooling Tower
Water Treatment Controller

ProMtrac_Browser_Manual.docx (5/23/13) rev1: – pn. 7501088

Please completely read through these operating instructions first! Do not discard! The warranty shall be invalidated by damage caused by operating errors!

ProMinent Fluid Controls, Inc. (USA) 136 Industry Drive, Pittsburgh, PA 15275

I/O Space    Wiring Navigation

Sidebar:

The physical connection points for inputs & outputs are designated

by letters (A-K) for inputs & 4-20mA outputs & numbers (1-5) for control relays

Using letters & numbers provides a way to refer to wiring points and I/O locations while

allowing users to change the names of these location to fit their site.

For Example: The bromine feed may be user labeled ‘Feeder 123’ but the wiring

location is always Relay No.3 & a ‘3’ tags the browser pages & the LCD displays

Login    Browser Navigation 1 of 3

#### On-Site using a Notebook PC to a ProMtrac not on the Site LAN

- You’ll need an Ethernet patch cable & a crossover adapter available

from office supply & electronics stores;     Example: Office Depot #942378,.

B.  You’ll need to set up a new connection in your notebook or PC.

Refer to Connect @ end of manual

Open the controller enclosure door, remove the lower access panel and jack into the controller Ethernet jack located @ the bottom center of the controller circuit board.

Start Internet Explorer or Mozilla’s Firefox.

Notebook PC & Over the Site LAN

Key the controller IP address into the PC’s browser address.

You can find the controller’s IP address using the controller keypad (default = 10.10.6.106).

Refer to Connect @ end of manual.

Login    Browser Navigation 2 of 3

Note: Views are optimized for limited resolution displays,

notebook & netbook  PCs at 1024 x 768 pixels.

Default Passwords:

The factory default userids & passwords are:

Operator1  = 1   Operator2  = 2   Operator3 = 3    Operator4 = 4.

Configure5 = 5  Configure6 = 6  Configure7 = 7    Administrator = AAAA

There are 3 password levels, Operator, Configure and Administrator.

The User IDs are used in the controller’s keypress log.

WARNING: 5 incorrect passwords, blocks logon until 7:00AM or until a power OFF/ON.

Login    Browser Navigation 2 of 3

The selector at the top, right side of the browser page navigates to both System & I/O pages.

System    Configure - Diagnostic

System    Ethernet

System    View Icons

System    Passwords

System    Time & Date

Sensors    Calibrate

Sensors    Configure     1 of 2

Sensors    Configure     2 of 2

Sensors    Alarms     1 of  2

Sensors    Alarms     2 of 2

Sensors    Diagnostic   1 of 2

Sensors    Diagnostic   2 of 2

Sensors    4-20mA Outputs

Controls    Auto / Prime / Stop

Controls    Setpoint   1 of 2

Controls    Setpoint   2 of 2

Controls    Configure  1 of 4

Controls    Configure  2 of 4

Controls    Configure  3 of 4

Controls    Configure  4 of 4

Controls    Alarms 1 of 3

Controls    Alarms 2 of 3

Controls    Alarms 3 of 3

Controls    Biocide Feed Events

Controls    Diagnostic

Connect    Ethernet Set-up  1 of 3, Windows

Windows operating systems have a simple way to find the Ethernet setup parameters:

You can use either the Keypad-LCD interface or the browser to setup the controller’s IP Address, Netmask, Gateway & Primary DNS.

Connect    Ethernet Set-up  2 of 3,   Windows 7

Connect    Ethernet Set-up  3 of 3,   Windows 7