Valuable Tips for Metering pumps

- Beta   SST, TTT and PVT4 (High viscosity) do not receive accessory kits.   Many customers assume they are receiving accessory kits

- Beta/b   Multiplier/divider is new.  Many won’t know what this is or how it can help them automate.

- Auto-degassing models are only suitable for suction lift applications.

- Integral bleed valve models work in both lift and flooded applications.  Once adjusted, bleed is automatic.

- Models with integral bleed valves include coarse adjustment for priming and fine adjustment for continuous degassing.

- PVT4, SST and TTT models do not have the option of an integral bleed valve.

- S1BA    Sigma/1 Basic. Pump comes with a motor as standard in the ident code.

- S2BA and S3Ba    Pump does not come with a motor.  Customer must purchase separately.

- Sigma Control versions   Motor Included

- Sigma 3 models: 070410, 070580, 040830 come with disk & spring valves, not balls.  Made of ceramic disks and Hastelloy C springs.

- Aside from a few customers, control cables are not included.  Required for external control of beta, gala, delta and sigma control models.

- Motor Driven pumps do not include Accessory Kits.  Must purchase FV and IV separately when required

- Customer must supply product serial number for the replacement of electronic cards.

- Delta Pumps are sold with both NPT and tube nozzle connections.

- Sigma 1 basic with 56c face motor adapter are for use with explosion proof motors only. Not for use with other motor types

- 99-1 Gear Ratio ProMus pumps are for use with size 17A pumps only

- ProMus pumps should not be sold for use with DC or VFD drives without consulting CSD

- ProMus pumps over 500 Psi are to be sold with double ball configuration

- HV models (PVT4) are single ball instead of the standard double ball valves.  Required for viscosities above 500 cps.  Recommended for flooded suction.

- Optionally, pumps can be ordered with 1 psi springs for products between 250-500 cps.  Easy to add or remove on site.